# Propaedeutic Digital Design

# “Basics of electronics”

Hi students,

There are two books available on internet for free that I use in this course and I recommend you widely:

- Digital Systems Principles and Applications. Ronald J. Tocci. 10th Ed. [1]

- Digital design With an Introduction to the Verilog HDL. F5th Ed. M. Morris Mano [2]

Please find help in those books and answer the following questions using your own words. Send your answers in private message today all day long.

- What is a computer?

- What is a program?

- Why we use digital computers instead analog computers? Give some advantages or disadvantages as you consider.

- Which semiconductor device is the basis of a digital component?

- Explain and draw the following circuits:

  - Turn on a LED using 87 VDC. Use the common working current of a LED of 20 mA.

  - Turn on 2 LEDs connected in parallel using 12 VDC.

  - Turn on as many LEDs as you can connected in series using 12VDC.

  - Turn on a LED using a common NPN bipolar transistor using a 12 VDC power supply for the LED and a 5 VDC as a trigger pulse; when the pulse is at the high level, the LED turns on and when the pulse is at low level, the LED is turned off.

  - Do the opposite working circuit as in the letter <d>; LED is ON when pulse is LOW and LED is OFF when pulse is HIGH. Same function as a NOT gate.

# BASICS OF COMBINATIONAL LOGIC CIRCUITS

Hi students,

Please answer the next problems in a word document (full_student_name.docx) and send it to me by private message. Revision date: June 22th all day long.

Please review the figure 3-16 of the book “Digital Systems Principles and Applications. Ronald J. Tocci” Tocci. 10th Ed.

- Draw the following circuits and get the truth table of each one:

- A’BC  + AB’C + ABC’

- A’B + AB’

- A XOR B XOR C

- Write the following expressions as Sum of Products (SOP) or Product of sums (POS) as appropriate.

- A’B + AB’

- (A+B)(A’+B’)

- A’B’C + A’BC’ + AB’C’ + ABC

- (A+B+C)(A+B’+C’)(A’+B+C’)(A’+B’+C)

- Program the Truth Table (TT) of any expression. The output will show up in the screen as in figure 3-16 or in figure 3-17. Considerations:

- The student chooses the programming language.

- It is widely recommendable that the student designs the algorithm using flow-charts before programming.

- The student chooses the number of input variables (A, B, C, etc.)

- The student chooses the input way and everything else as necessary. For instance, If the input will be whether in POS or SOP.

Figure 3-17 Example of an output of a code in C

# Combinational Logic

On the next link, you will be able to find both books [1] and [2].:

https://www.dropbox.com/sh/gtfydeqe8vk5j5n/AACT3-NILCxtx3V-IVY5nDQpa?dl=0

Please read the section related to decoders and solve the next exercises in a word document (full_student_name_4.docx) and send it to me by private message:

- Get the truth table of a common anode 7-segment display

- Get the truth table of a common cathode7-segment display.

- Get the output equation (Anode and Cathode) of each segment: A, B, C, D, E, F, G and DP.

- For each input code <X3:X0> give the output in hexadecimal <A:DP>|

| Display | Inputs |  |  |  |  | Outputs Cathode |  |  |  |  |  |  |  | HEX |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number | x3 | x2 | x1 | x0 |  | a | b | c | d | e | f | g | dot |  |
| 0 | 0 | 0 | 0 | 0 |  | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0XFC |
| 1 | 0 | 0 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |
| 2 | 0 | 0 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |
| 3 | 0 | 0 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |
| 4 | 0 | 1 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |
| 5 | 0 | 1 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |
| 6 | 0 | 1 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |
| 7 | 0 | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |
| 8 | 1 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |
| 9 | 1 | 0 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |
| A | 1 | 0 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |
| B | 1 | 0 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |
| C | 1 | 1 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |
| D | 1 | 1 | 0 | 1 |  |  |  |  |  |  |  |  |  |  |
| E | 1 | 1 | 1 | 0 |  |  |  |  |  |  |  |  |  |  |
| F | 1 | 1 | 1 | 1 |  |  |  |  |  |  |  |  |  |  |

# Multiplexers

- Please read and describe using your own words all the elements that you need in order to create a 8:1 Multiplexer. Draw the Logic Diagram and Write the output equation.

- Implement the next Boolean function using a multiplexer:

X(A,B,C,D) = Minterms(0, 4, 5, 9, 10, 11, 14, 15)

Get the equation of “X”.

Note: Please Respect the notation (The variables A, B, C, D, X) as indicated.

Hi everyone,

Please read and do the exercises related to “Multiplexers” of the section 4 of the document “Digital Design propaedeutic.docx” The document is available on the link that I gave you previously.

These exercises are a requirement for your first exam. Please do them.

Revision date: Today –Friday, July 3, 2020 all day long.

Exam date: Monday, July 6, 2020 at 15h00. Duration: 02h00. Please study.

# Exam

Instructions: please answer the following questions in a word document (No images, No PDF), name it as “full_student_name_exam1.docx” and send it to me as private message.

Important: Do not change any variable name.

- Suppose that you have a power supply of 12VDC. Design just one Resistor-array to get 3VDC, 5VDC, and 9VDC.

- Get the truth table, the reduced equation in SOP form and the circuit of the function:

X(A, B, C, D, E) = Minterms(0, 5, 6, 9, 11, 12, 15)

- Implement the next Boolean function using a multiplexer and draw the final circuit using the multiplexer:

X(A,B,C,D) = Maxterms(1, 2, 3, 7, 11, 12, 14)

Get the equation of “X” in POS form.

A FLIP-FLOP is a storage element that works with pulses. See Table 5.1 and figure 5.11.

- Get the equation of each Multiplexer shown in figure 6.7

0 = MEMORY

1 = SHIFT RIGHT

2 = SHIFT LEFT

3 = PARALELL LOAD

MUX0 = S1’ S0’ A0 + S1’ S0 A1 + S1 S0’ L + S1 S0 I0

MUX1 = S1’ S0’ A1 + S1’ S0 A2 + S1 S0’ A0 + S1 S0 I1

MUX2 = S1’ S0’ A2 + S1’ S0 A3 + S1 S0’ A1 + S1 S0 I2

MUX2 = S1’ S0’ A3 + S1’ S0 R +  S1 S0’ A2 + S1 S0 I3

# Flip Flop

- From Flip-Flop D, get the truth table / equation of JK and T Flip-Flops. See figures from 5.11 and so on.

- Design an upward synchronous counter like 0,1,2,3,4,5,6,7 and repeat again from 0.

Use D Flip-Flop.

Revision Date: Wednesday July 8, 2020 before 18h00.

Send it to me as “Full_name_FlipFlop.docx”

# Final Exam

Please Get the reduced equations of each bit of the following situation:

Taking “X” as selector and using FF-D:

If X = ‘0’, count upward from 0, 1, 2, 3, 4, 5, 6, 7 and repeat.

If X = ‘1’, count upward from 1, 2, 3, 8, 9, 15 and repeat.

Note: write B0 as the least significant bit. Use as many bits as necessary, for instance:

<Bn, Bn-1 …, B1, B0>

Revision date: Today Friday, July 10, 2020. The latest at 20h30.

Send me your exam as private message>  “full_name_finalExam.docx”.